Public Transport  Victoria

Yarra Trams Load Standards Survey Report

May 2014

Table of Contents

1	Summary and Key Results	3

1.1	Introduction	3

1.2	Key Results	3

2	Background and Definitions	4

2.1	Background	4

2.2	Data Collection and Analysis	4

2.3	Terminology	5

3	Results	6

3.1	Rolling hour average loads above and within 10% of desired standards	6

3.1.1	Key Results	15

3.2	Rolling hour average loads above desired standards – Cordon locations	16

3.2.1	Key Results	16

3.3	Average Daily Observed Loads	17

3.3.1	Key Results	18

Appendix 1	Map of survey locations	19

Appendix 2	List of survey locations	20

Appendix 3	Locations where routes are combined into corridors	22

Appendix 4	Maximum capacities by tram type	23

Appendix 5	Comparison of May 2013 and May 2014 average load and average maximum capacity rolling hour results at locations with breaches in May 2014	24

Appendix 6	Charts of services observed in rolling hours where load is more than 90 per cent of capacity available	57

# Summary and Key Results

## Introduction

The Metropolitan Tram Load Standards Surveys are conducted twice yearly, in May and October, to measure passenger loads against desired load standards.

The survey’s findings help pinpoint at which sections on each route and at which times loads exceed the desired standards.  Surveying is undertaken during the AM and PM peaks.

This bulletin reports on the May 2014 survey which was conducted from 5 May to 27 May 2014

Note: See Section 2 Background and Definitions for explanations on routes and corridors.

## Key Results

Table 1 Rolling hour average loads above desired standards recorded in May 2014 survey

|  | Number of Rolling Hour Average Loads above desired standards | Number of Routes | Number of Corridors |
| --- | --- | --- | --- |
| AM Peak | 27 | 6 | 3 |
| PM Peak | 14 | 5 | 4 |
| Total | 41 |  |  |

In May 2014, a total of 41 rolling hour average loads above desired standards were recorded, 27 of those were observed in the AM peak, and 14 in the PM peak.

During the May 2014 Load Standard Survey, a total of 28 locations were surveyed while 39 locations were surveyed in May 2014.

At locations surveyed in 2014 there were a total of 41 rolling hour average loads above desired standards; 27 in the AM peak and 14 in the PM peak.

When comparing locations observed in both the May 2013 and May 2014 Load Standards Surveys, there was an increase from 32 to 39 rolling hour average loads above desired standards.

When comparing cordon locations observed in both the May 2013 and May 2014 Load Standards Surveys, there was an increase from 22 to 26 rolling hour average loads above desired standards.

# Background and Definitions

## Background

Metropolitan Tram Passenger Load Standards Surveys have been conducted since 2000 as part of the Franchise Agreement (the Agreement) between the metropolitan tram operator and the State Government.

Up until 2009, the surveys were conducted annually in October. A second yearly survey was introduced from May 2009.  Traditionally the October survey has been larger in scope, however in 2011 the decision was made to increase the May survey so that it could inform the annual Strategic Operations Plan.

When developing the survey, locations previously recording load breaches or rolling hour average loads close to breach are included.  In addition, locations impacted by service changes either to the tram or train timetables may be included.

The survey results have an important role in the examination of the allocation of tram capacity across the network.

## Data Collection and Analysis

Data was collected over weekdays between 5 May to 27 May 2014, by independent surveyors from a contracted labour supply company.

Passenger counts were undertaken between 7.00 am and 10.00 am and between 3.00 pm and 7.00 pm at selected tram stops across the network, with all services under the Master Timetable surveyed.

In accordance with the Agreement each survey location was surveyed at least three times, on different weekdays, to form a Sampled Service Set.

At most locations passenger counts were conducted in the direction of heading into the city in the morning and heading out of the city in the evening. However, at specific locations where high bi-directional passenger loads are known to occur surveying was conducted in both directions.

Between one and six surveyors counted loads at each tram stop, depending on the frequency and volume of services at the stop. If more than one surveyor was present, surveyors worked in pairs with one recording the information and the other conducting the passenger count.

The surveyors recorded the tram number, route number, arrival time and arrival or departure load. Arrival loads were recorded for morning shifts and departure loads for evening shifts, except at selected locations as specified by stakeholders, where exceptions to this rule applied. (See Appendix 2 on page 14 for more details)

On lightly loaded trams, each passenger was counted. On heavily loaded trams, the number of people standing and the empty seats were counted. The number of standing passengers was added to the known number of seats on the tram and the number of empty seats subtracted. Information on the seating capacity of each tram type is included in the table at Appendix 4.

Most routes passing through the CBD, and other routes that share substantial sections of track, have been combined into corridors. A list of the routes aggregated into corridors is at Appendix 3.

The average load for a route or corridor at each survey location was calculated by adding together the total number of passengers on each service surveyed in a rolling hour, and dividing that by the number of services in that rolling hour. The average passenger loads are then measured against desired load standards.

The average maximum capacity is calculated by totalling the individual maximum capacities of each service on each route and dividing that by the number of services for each rolling hour. Maximum capacity for each tram class is considered greater at CBD locations than at non-CBD locations (refer to Appendix 4).

## Terminology

AM Peak – the period on a weekday between 7.31 am and 9.30 am. The period of surveying for the May 2013 survey, 7.00 am to 10.00 am, included the peak and an extra half-hour before and after the peak.

Corridor – the aggregation of Rolling Hour survey results is performed across tram routes at locations where the routes share substantial sections of track and travel to the same destination.

CBD monitoring point – means any point in the Central Business District (CBD), or any other point determined by the Director, at which passenger counts are conducted in accordance with the Agreement.

Load – the number of passengers travelling on a tram.

Location – a generic term indicating a monitoring point, with CBD or Non CBD as context requires.

Maximum capacity – the desired maximum capacity of a tram in service. (See Appendix 4 for details of capacities for each tram class). The maximum capacity is higher for trams operating in the CBD than for trams operating outside the CBD. This is also referred to as desired standard in this report. This standard does not relate to a safety requirement but to the relative comfort level.

Non-CBD monitoring point – means any point outside the CBD at which passenger counts are conducted in accordance with the Agreement.

PM Peak – the period on a weekday between 3.31 pm and 6.30 pm. The period of surveying for the May 2012 survey, 3.00 pm to 7.00 pm, included the peak and an extra half-hour before and after the peak.

Peak – either or both the AM Peak and the PM Peak, as the context requires.

Rolling hour – each hour from 6.01am to 12 midnight, commencing on each hour and each half-hour during that period.

Survey direction – the direction of travel of services that are to be included in passenger counts conducted in accordance with the Agreement.

# Results

## Rolling hour average loads above and within 10% of desired standards

There were 41 observed rolling hour average loads above desired standards in the May 2014 survey.

- Of these, 27 were during the AM peak and 14 during the PM peak.

- In addition, there were 36 rolling hour average loads within ten per cent of desired standard, 18 in the AM peak, and 18 in the PM peak.

Table 2 below shows a comparison of the monitoring points (locations) at which rolling hour average loads exceeded or were within 10% desired standards in either May 2013 or May 2014.

Note: Locations falling within these criteria in May 2013, but not surveyed in May 2014 were not included in this table.

Table 2 Comparison of routes and locations/directions with rolling hour average loads above and within 10% desired standards, May 2013 and 2014

Key:

*	Below 80% of Load Standard

@	Within 20% of Load Standard

#	Within 10% of Load Standard

!	Load Standard Breach

| Location | Route | Direction | Rolling Hour | Rolling Hour Average Load 2013 | Average Maximum Capacity 2013 | % Average Maximum Capacity 2013 | Rolling Hour Average Load 2014 | Average Maximum Capacity 2014 | % Average Maximum Capacity 2014 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-CBD Locations - AM Peak |  |  |  |  |  |  |  |  |  |
| Dandenong Rd / Hornby St | 5, 64 | West Bound | 7:31-8:30 | 68.7 | 65.6 | !104.8% | 69.4 | 68.0 | !102.1% |
|  |  |  | 8:01-9:00 | 73.2 | 64.1 | !114.3% | 69.7 | 61.3 | !113.8% |
| Dawson St/Daly St | 55 | South Bound | 7:01-8:00 | -- | -- | -- | 60.6 | 73.0 | @83.1% |
|  |  |  | 7:31-8:30 | -- | -- | -- | 73.1 | 78.1 | #93.6% |
|  |  |  | 8:01-9:00 | -- | -- | -- | 60.4 | 66.4 | #91.0% |
| Elgin St / Lygon St | 1, 8 | South Bound | 7:31-8:30 | 70.8 | 77.6 | #91.3% | 78.6 | 76.6 | !102.7% |
|  |  |  | 8:01-9:00 | 88.6 | 81.6 | !108.6% | 96.3 | 77.6 | !124.1% |
|  |  |  | 8:31-9:30 | 81.0 | 74.5 | !108.7% | 83.3 | 68.8 | !121.0% |
|  |  |  | 9:01-10:00 | 60.9 | 67.9 | @89.7% | 67.7 | 66.3 | !102.0% |
| Errol St / Victoria St | 57 | South Bound | 7:31-8:30 | 60.1 | 60.0 | !100.2% | 51.2 | 60.0 | @85.3% |
|  |  |  | 8:01-9:00 | 63.3 | 60.0 | !105.5% | 52.8 | 60.0 | @87.9% |
|  |  |  | 8:31-9:30 | 51.6 | 60.0 | @86.0% | 47.3 | 60.0 | *78.8% |
| Flemington Rd / Abbotsford St | 55 | South Bound | 7:01-8:00 | 52.8 | 68.2 | *77.4% | 65.9 | 71.3 | #92.5% |
|  |  |  | 7:31-8:30 | 75.2 | 78.1 | #96.3% | 87.2 | 80.5 | !108.3% |
|  |  |  | 8:01-9:00 | 80.8 | 72.4 | !111.5% | 81.1 | 71.6 | !113.3% |
|  |  |  | 8:31-9:30 | 67.1 | 65.4 | !102.6% | 65.6 | 64.2 | !102.1% |
|  | 59 | South Bound | 7:01-8:00 | 73.7 | 100.0 | *73.7% | 87.4 | 100.0 | @87.4% |
|  |  |  | 7:31-8:30 | 88.0 | 100.0 | @88.0% | 101.4 | 100.0 | !101.4% |
|  |  |  | 8:01-9:00 | 93.9 | 100.0 | #93.9% | 108.2 | 98.9 | !109.4% |
|  |  |  | 8:31-9:30 | 98.0 | 100.0 | #98.0% | 108.1 | 98.6 | !109.6% |
|  |  |  | 9:01-10:00 | 93.6 | 100.0 | #93.6% | 95.3 | 100.0 | #95.3% |
| Grantham St / Brunswick Rd | 55 | South Bound | 7:01-8:00 | 61.8 | 71.7 | @86.3% | 61.0 | 72.6 | @84.1% |
|  |  |  | 7:31-8:30 | 80.2 | 77.8 | !103.1% | 74.1 | 76.7 | #96.6% |
|  |  |  | 8:01-9:00 | 76.0 | 69.2 | !109.8% | 64.2 | 67.1 | #95.7% |
|  |  |  | 8:31-9:30 | 61.0 | 65.0 | #93.8% | 56.2 | 66.9 | @84.1% |
| Haymarket (Elizabeth St) | 19 | South Bound | 8:01-9:00 | 80.7 | 103.8 | *77.8% | 90.2 | 105.3 | @85.7% |
|  |  |  | 8:31-9:30 | 78.1 | 103.4 | *75.5% | 94.5 | 105.0 | @90.0% |
|  | 59 | South Bound | 7:31-8:30 | 75.2 | 100.0 | *75.2% | 86.9 | 100.0 | @86.9% |
|  |  |  | 8:01-9:00 | 77.7 | 100.0 | *77.7% | 97.4 | 100.0 | #97.4% |
|  |  |  | 8:31-9:30 | 77.6 | 100.0 | *77.6% | 105.7 | 98.7 | !107.1% |
|  |  |  | 9:01-10:00 | 72.0 | 100.0 | *72.0% | 100.4 | 98.4 | !102.0% |
| Peel St / Victoria St | 55 | South Bound | 7:01-8:00 | 54.9 | 68.0 | @80.7% | 60.8 | 68.6 | @88.6% |
|  |  |  | 7:31-8:30 | 73.7 | 78.0 | #94.5% | 86.6 | 79.5 | !108.9% |
|  |  |  | 8:01-9:00 | 80.6 | 74.0 | !108.9% | 86.6 | 74.5 | !116.2% |
|  |  |  | 8:31-9:30 | 64.9 | 64.3 | !101.0% | 67.6 | 63.0 | !107.3% |
|  |  |  | 9:01-10:00 | 50.1 | 71.0 | *70.6% | 59.4 | 71.6 | @83.0% |
| Queensbridge St (Casino East) | 55 | North Bound | 7:31-8:30 | 55.1 | 64.8 | @85.1% | 58.0 | 65.0 | @89.2% |
|  |  |  | 8:01-9:00 | 53.2 | 76.4 | *69.6% | 66.1 | 75.6 | @87.4% |
| Royal Pde / Grattan St (Royal Melbourne Hospital) | 19 | South Bound | 8:01-9:00 | 75.2 | 103.6 | *72.6% | 88.5 | 105.3 | @84.1% |
|  |  |  | 8:31-9:30 | 73.8 | 103.2 | *71.5% | 91.3 | 105.0 | @86.9% |
| South Melbourne Market (South Melbourne LRS) | 96 | North Bound | 7:31-8:30 | 110.4 | 121.8 | #90.7% | 112.5 | 138.2 | @81.4% |
|  |  |  | 8:01-9:00 | 117.1 | 121.3 | #96.5% | 114.9 | 135.2 | @85.0% |
| St Kilda Rd / Commercial Rd | 3, 5, 16, 64, 67 | North Bound | 7:31-8:30 | 61.0 | 69.7 | @87.5% | 67.6 | 70.2 | #96.2% |
|  |  |  | 8:01-9:00 | 71.9 | 63.4 | !113.5% | 70.8 | 62.6 | !113.0% |
|  |  |  | 8:31-9:30 | 53.9 | 63.1 | @85.4% | 53.8 | 62.3 | @86.4% |
|  | 6 | North Bound | 7:31-8:30 | 57.2 | 77.8 | *73.5% | 64.2 | 78.9 | @81.3% |
|  |  |  | 8:01-9:00 | 62.8 | 76.7 | @81.9% | 65.6 | 76.7 | @85.6% |
|  | 72 | North Bound | 7:31-8:30 | 47.3 | 72.5 | *65.3% | 59.7 | 72.9 | @82.0% |
|  |  |  | 8:01-9:00 | 57.0 | 66.2 | @86.2% | 60.3 | 69.2 | @87.1% |
| St Kilda Rd / High St | 3, 5, 16, 64, 67 | North Bound | 7:31-8:30 | 61.0 | 68.9 | @88.6% | 68.1 | 69.4 | #98.1% |
|  |  |  | 8:01-9:00 | 74.4 | 63.0 | !118.0% | 73.2 | 61.9 | !118.2% |
|  |  |  | 8:31-9:30 | 56.8 | 63.5 | @89.5% | 52.8 | 62.7 | @84.3% |
|  | 6 | North Bound | 7:31-8:30 | 59.7 | 77.8 | *76.7% | 65.5 | 78.9 | @82.9% |
|  |  |  | 8:01-9:00 | 62.7 | 75.8 | @82.8% | 63.9 | 75.8 | @84.3% |
| St Vincents Plaza | 24 | West Bound | 7:31-8:30 | 50.1 | 64.0 | *78.2% | 56.3 | 64.6 | @87.1% |
|  |  |  | 8:01-9:00 | 52.7 | 64.0 | @82.3% | 64.3 | 71.1 | #90.5% |
|  |  |  | 8:31-9:30 | 46.2 | 62.5 | *73.9% | 61.8 | 70.0 | @88.3% |
|  | 109 | West Bound | 8:01-9:00 | 77.5 | 96.8 | @80.1% | 80.5 | 94.8 | @84.9% |
|  | 112 | West Bound | 7:31-8:30 | 58.7 | 73.3 | @80.2% | 68.4 | 71.6 | #95.6% |
|  |  |  | 8:01-9:00 | 70.2 | 80.6 | @87.1% | 75.8 | 69.9 | !108.4% |
|  |  |  | 8:31-9:30 | 66.2 | 73.6 | @89.9% | 71.8 | 66.5 | !108.0% |
|  |  |  | 9:01-10:00 | 57.0 | 62.1 | #91.8% | 56.2 | 65.0 | @86.4% |
| Toorak Rd / Punt Rd | 8 | West Bound | 7:31-8:30 | -- | -- | -- | 78.9 | 68.0 | !116.1% |
|  |  |  | 8:01-9:00 | -- | -- | -- | 73.9 | 68.9 | !107.2% |
| Victoria St/Peel St | 57 | South Bound | 7:31-8:30 | -- | -- | -- | 50.0 | 60.0 | @83.4% |
|  |  |  | 8:01-9:00 | -- | -- | -- | 53.3 | 60.0 | @88.8% |
|  |  |  | 8:31-9:30 | -- | -- | -- | 51.7 | 60.0 | @86.2% |
| Wellington Pde / Jolimont Rd | 48 | West Bound | 7:01-8:00 | 57.2 | 57.1 | !100.1% | 59.3 | 67.9 | @87.4% |
|  |  |  | 7:31-8:30 | 69.6 | 56.6 | !123.0% | 73.4 | 72.7 | !101.0% |
|  |  |  | 8:01-9:00 | 80.5 | 61.0 | !132.0% | 80.1 | 71.5 | !112.0% |
|  |  |  | 8:31-9:30 | 75.8 | 62.5 | !121.2% | 72.8 | 65.4 | !111.4% |
|  |  |  | 9:01-10:00 | 62.3 | 57.4 | !108.6% | 55.0 | 64.8 | @84.9% |
|  | 75 | West Bound | 7:31-8:30 | 79.5 | 98.0 | @81.1% | 94.6 | 95.9 | #98.6% |
|  |  |  | 8:01-9:00 | 88.7 | 96.3 | #92.2% | 102.2 | 96.3 | !106.2% |
|  |  |  | 8:31-9:30 | 78.4 | 95.5 | @82.0% | 95.6 | 100.0 | #95.6% |
| CBD Locations - AM Peak |  |  |  |  |  |  |  |  |  |
| Clarendon St (Crown Casino) | 96 | North Bound | 8:01-9:00 | 114.4 | 132.6 | @86.3% | 127.9 | 145.2 | @88.1% |
|  | 109 | North Bound | 8:01-9:00 | 81.3 | 108.7 | *74.8% | 88.2 | 105.6 | @83.5% |
|  | 112 | North Bound | 7:31-8:30 | 55.3 | 65.0 | @85.1% | 66.2 | 77.9 | @85.0% |
|  |  |  | 8:01-9:00 | 65.4 | 65.0 | !100.7% | 86.3 | 88.1 | #98.0% |
|  |  |  | 8:31-9:30 | 47.6 | 75.4 | *63.1% | 62.1 | 71.9 | @86.4% |
| Collins St West End (Southern Cross Station) | All routes (CBD) | West Bound | 8:01-9:00 | 65.0 | 79.3 | @81.9% | 65.0 | 84.1 | *77.3% |
|  |  |  | 8:31-9:30 | 62.9 | 78.4 | @80.2% | 58.6 | 77.3 | *75.9% |
| Swanston St / Flinders St (Federation Sq) | 1 | North Bound | 7:31-8:30 | 58.6 | 82.8 | *70.7% | 65.4 | 75.3 | 86.8% |
|  |  |  | 8:01-9:00 | 62.4 | 80.5 | *77.5% | 64.5 | 75.9 | @84.9% |
|  | 3, 5, 6, 16, 64, 67, 72 | South Bound | 7:31-8:30 | 82.2 | 79.7 | !103.1% | 76.0 | 79.8 | #95.2% |
|  |  |  | 8:01-9:00 | 94.7 | 83.6 | !113.4% | 83.6 | 84.3 | #99.2% |
|  |  |  | 8:31-9:30 | 66.3 | 77.5 | @85.5% | 62.3 | 78.6 | 79.2% |
|  | 1 | South Bound | 7:31-8:30 | 72.4 | 82.6 | @87.6% | 68.9 | 84.0 | @82.0% |
|  |  |  | 8:01-9:00 | 88.5 | 92.0 | #96.2% | 90.8 | 90.9 | #99.9% |
|  |  |  | 8:31-9:30 | 65.4 | 92.1 | *71.0% | 80.4 | 88.3 | #91.0% |
|  | 8 | South Bound | 8:01-9:00 | 71.6 | 84.2 | @85.1% | 54.7 | 82.3 | 66.4% |
| Non-CBD Locations - PM Peak |  |  |  |  |  |  |  |  |  |
| Dandenong Rd / Hornby St | 5, 64 | East Bound | 17:01-18:00 | 60.1 | 65.5 | #91.8% | 60.8 | 65.0 | #93.6% |
|  |  |  | 17:31-18:30 | 61.8 | 69.7 | @88.7% | 67.7 | 64.4 | !105.1% |
|  |  |  | 18:01-19:00 | 54.3 | 70.8 | *76.7% | 53.7 | 65.0 | @82.6% |
| Dawson St/Daly St | 55 | North Bound | 15:01-16:00 | -- | -- | -- | 53.1 | 62.2 | @85.3% |
| Elgin St / Lygon St | 1, 8 | North Bound | 16:31-17:30 | 57.9 | 74.1 | *78.2% | 66.8 | 71.8 | #93.0% |
|  |  |  | 17:01-18:00 | 71.1 | 73.5 | #96.7% | 78.7 | 75.3 | !104.5% |
|  |  |  | 17:31-18:30 | 81.5 | 76.7 | !106.3% | 82.7 | 73.3 | !112.8% |
|  |  |  | 18:01-19:00 | 72.6 | 78.4 | #92.5% | 83.0 | 72.3 | !114.8% |
| Errol St / Victoria St | 57 | North Bound | 18:01-19:00 | 50.1 | 60.0 | @83.4% | 45.3 | 60.0 | *75.5% |
| Flemington Rd / Abbotsford St | 55 | North Bound | 15:01-16:00 | 51.6 | 68.4 | *75.5% | 55.8 | 68.9 | @81.0% |
|  |  |  | 16:31-17:30 | 65.2 | 77.6 | @84.0% | 67.5 | 73.8 | #91.6% |
|  |  |  | 17:01-18:00 | 73.4 | 71.7 | !102.3% | 72.8 | 72.2 | !100.8% |
|  |  |  | 17:31-18:30 | 72.7 | 75.8 | #95.9% | 76.1 | 79.5 | #95.7% |
|  |  |  | 18:01-19:00 | 68.8 | 85.7 | @80.3% | 68.3 | 82.7 | @82.6% |
|  | 59 | North Bound | 15:01-16:00 | 61.0 | 100.0 | *61.0% | 83.6 | 100.0 | @83.6% |
|  |  |  | 16:31-17:30 | 77.1 | 100.0 | *77.1% | 97.7 | 100.0 | #97.7% |
|  |  |  | 17:01-18:00 | 84.6 | 100.0 | @84.6% | 110.6 | 100.0 | !110.6% |
|  |  |  | 17:31-18:30 | 87.5 | 100.0 | @87.5% | 97.2 | 100.0 | #97.2% |
|  |  |  | 18:01-19:00 | 78.4 | 100.0 | *78.4% | 89.0 | 100.0 | @89.0% |
| Grantham St / Brunswick Rd | 55 | North Bound | 15:01-16:00 | 41.4 | 68.0 | *60.8% | 55.8 | 68.0 | @82.1% |
|  |  |  | 15:31-16:30 | 46.2 | 69.5 | *66.5% | 57.8 | 70.4 | @82.1% |
|  |  |  | 17:01-18:00 | 55.9 | 73.3 | *76.2% | 64.7 | 71.6 | #90.3% |
|  |  |  | 17:31-18:30 | 62.9 | 75.4 | @83.4% | 68.9 | 76.8 | @89.7% |
|  |  |  | 18:01-19:00 | 65.4 | 80.7 | @81.0% | 66.1 | 78.1 | @84.6% |
| Haymarket (Elizabeth St) | 19 | North Bound | 17:01-18:00 | 95.5 | 103.5 | #92.3% | 87.8 | 106.1 | @82.7% |
|  |  |  | 17:31-18:30 | 92.6 | 102.4 | #90.5% | 89.8 | 106.7 | @84.2% |
|  |  |  | 18:01-19:00 | 82.0 | 101.9 | @80.5% | 76.4 | 104.0 | *73.5% |
|  | 59 | North Bound | 16:31-17:30 | 80.4 | 100.0 | @80.4% | 73.7 | 100.0 | *73.7% |
|  |  |  | 17:01-18:00 | 86.1 | 100.0 | @86.1% | 92.6 | 100.0 | #92.6% |
|  |  |  | 17:31-18:30 | 79.8 | 100.0 | *79.8% | 86.7 | 100.0 | @86.7% |
|  |  |  | 18:01-19:00 | 83.3 | 100.0 | @83.3% | 80.0 | 100.0 | @80.0% |
| Peel St / Victoria St | 55 | North Bound | 17:01-18:00 | 67.6 | 72.2 | #93.5% | 68.4 | 76.0 | @89.9% |
|  |  |  | 17:31-18:30 | 69.5 | 78.2 | @88.9% | 72.8 | 81.6 | @89.2% |
|  |  |  | 18:01-19:00 | 59.1 | 83.7 | *70.7% | 63.3 | 78.5 | @80.7% |
| Royal Pde / Grattan St (Royal Melbourne Hospital) | 19 | North Bound | 16:31-17:30 | 81.0 | 102.9 | *78.7% | 84.1 | 104.2 | @80.6% |
|  |  |  | 17:01-18:00 | 99.8 | 104.4 | #95.6% | 100.3 | 105.7 | #94.9% |
|  |  |  | 17:31-18:30 | 92.6 | 103.0 | @89.9% | 98.8 | 107.0 | #92.3% |
|  |  |  | 18:01-19:00 | 82.5 | 102.9 | @80.2% | 83.3 | 104.0 | @80.1% |
| South Melbourne Market (South Melbourne LRS) | 96 | South Bound | 17:01-18:00 | 99.0 | 122.0 | @81.2% | 99.1 | 126.5 | *78.4% |
|  |  |  | 17:31-18:30 | 110.4 | 122.3 | #90.3% | 102.7 | 131.5 | *78.1% |
| St Kilda Rd / Commercial Rd | 3, 5, 16, 64, 67 | South Bound | 17:01-18:00 | 59.7 | 67.8 | @88.1% | 59.9 | 67.2 | @89.1% |
|  |  |  | 17:31-18:30 | 57.7 | 71.5 | @80.7% | 59.0 | 69.2 | @85.3% |
|  | 6 | South Bound | 17:01-18:00 | 52.2 | 63.0 | @82.8% | 54.1 | 65.7 | @82.4% |
|  | 72 | South Bound | 17:01-18:00 | 67.4 | 73.8 | #91.4% | 58.5 | 76.3 | *76.7% |
| St Kilda Rd / High St | 3, 5, 16, 64, 67 | South Bound | 17:01-18:00 | 60.4 | 66.4 | #90.9% | 60.1 | 67.3 | @89.3% |
|  |  |  | 17:31-18:30 | 59.2 | 71.8 | @82.5% | 56.5 | 69.2 | @81.6% |
|  | 6 | South Bound | 17:01-18:00 | 59.8 | 63.0 | #94.9% | 57.3 | 66.3 | @86.4% |
|  |  |  | 17:31-18:30 | 56.8 | 64.8 | @87.6% | 53.8 | 66.7 | @80.7% |
| St Vincents Plaza | 109 | East Bound | 16:31-17:30 | 70.1 | 98.3 | *71.3% | 81.3 | 96.3 | @84.4% |
|  |  |  | 17:01-18:00 | 68.3 | 97.8 | *69.8% | 77.2 | 94.4 | @81.8% |
|  | 112 | East Bound | 15:31-16:30 | 56.3 | 60.0 | #93.9% | 50.7 | 73.0 | *69.3% |
|  |  |  | 17:01-18:00 | 63.2 | 68.0 | #92.9% | 63.2 | 72.1 | @87.6% |
|  |  |  | 17:31-18:30 | 62.6 | 66.3 | #94.4% | 63.0 | 70.6 | @89.3% |
|  |  |  | 18:01-19:00 | 62.5 | 65.8 | #95.0% | 55.8 | 76.4 | *73.0% |
| Swan St / Punt Rd | 70 | East Bound | 17:31-18:30 | 57.5 | 57.3 | !100.3% | 42.5 | 60.4 | *70.3% |
|  |  |  | 18:01-19:00 | 49.5 | 55.0 | @89.9% | 35.7 | 59.5 | *59.9% |
| Toorak Rd / Punt Rd | 8 | East Bound | 15:01-16:00 | -- | -- | -- | 59.7 | 70.5 | @84.7% |
| Victoria St/Peel St | 57 | North Bound | 17:01-18:00 | -- | -- | -- | 52.2 | 60.0 | @87.1% |
|  |  |  | 17:31-18:30 | -- | -- | -- | 56.5 | 60.0 | #94.1% |
|  |  |  | 18:01-19:00 | -- | -- | -- | 57.4 | 60.0 | #95.7% |
| Wellington Pde / Jolimont Rd | 48 | East Bound | 16:01-17:00 | 48.4 | 55.0 | @88.0% | 55.4 | 61.4 | #90.2% |
|  |  |  | 16:31-17:30 | 57.2 | 55.0 | !104.0% | 65.0 | 60.9 | !106.8% |
|  |  |  | 17:01-18:00 | 65.3 | 58.2 | !112.1% | 71.8 | 72.1 | #99.7% |
|  |  |  | 17:31-18:30 | 64.7 | 58.0 | !111.6% | 76.2 | 73.0 | !104.3% |
|  |  |  | 18:01-19:00 | 64.6 | 55.0 | !117.4% | 76.3 | 57.0 | !133.8% |
|  | 75 | East Bound | 17:01-18:00 | 79.8 | 96.1 | @83.1% | 96.1 | 94.7 | !101.4% |
|  |  |  | 17:31-18:30 | 80.5 | 93.6 | @86.1% | 88.0 | 91.8 | #95.8% |
| CBD Locations - PM Peak |  |  |  |  |  |  |  |  |  |
| Clarendon St (Crown Casino) | 96 | South Bound | 17:01-18:00 | 111.4 | 133.9 | @83.2% | 107.6 | 135.8 | *79.2% |
|  |  |  | 17:31-18:30 | 115.0 | 133.9 | @85.9% | 106.8 | 142.0 | *75.2% |
|  | 112 | South Bound | 17:01-18:00 | 55.1 | 68.8 | @80.2% | 64.0 | 83.9 | *76.3% |
|  |  |  | 17:31-18:30 | 61.0 | 74.0 | @82.4% | 58.1 | 72.8 | *79.8% |
| Collins St West End (Southern Cross Station) | All routes (CBD) | East Bound | 15:31-16:30 | 64.5 | 68.5 | #94.3% | 57.7 | 71.9 | @80.3% |
|  |  |  | 16:01-17:00 | 59.9 | 73.7 | @81.2% | 57.6 | 77.7 | *74.1% |
|  |  |  | 16:31-17:30 | 65.0 | 71.7 | #90.7% | 71.0 | 76.6 | #92.7% |
|  |  |  | 17:01-18:00 | 76.9 | 68.3 | !112.5% | 83.1 | 81.6 | !101.8% |
|  |  |  | 17:31-18:30 | 81.3 | 67.4 | !120.6% | 75.4 | 80.9 | #93.2% |
|  |  |  | 18:01-19:00 | 68.1 | 65.0 | !104.8% | 58.5 | 73.5 | *79.5% |
| Swanston St / Flinders St (Federation Sq) | 3, 5, 6, 16, 64, 67, 72 | North Bound | 16:31-17:30 | 63.5 | 79.5 | *79.8% | 67.9 | 79.3 | @85.7% |
|  |  |  | 17:01-18:00 | 76.2 | 81.5 | #93.5% | 81.0 | 80.0 | !101.3% |
|  |  |  | 17:31-18:30 | 71.4 | 78.2 | #91.3% | 80.1 | 77.5 | !103.4% |
|  | 1 | North Bound | 16:31-17:30 | 71.4 | 82.5 | @86.6% | 80.1 | 80.4 | #99.6% |
|  |  |  | 17:01-18:00 | 80.1 | 89.0 | @89.9% | 97.9 | 90.5 | !108.2% |
|  |  |  | 17:31-18:30 | 81.7 | 100.6 | @81.2% | 77.5 | 97.2 | *79.7% |
|  | 8 | North Bound | 17:01-18:00 | 73.3 | 83.6 | @87.6% | 64.7 | 83.2 | *77.8% |
|  |  |  | 17:31-18:30 | 48.4 | 74.0 | *65.3% | 61.6 | 73.9 | @83.4% |

### Key Results

- When comparing locations observed in both the May 2013 and May 2014 Load Standards Surveys, there was an increase from 32 to 39 rolling hour average loads above desired standards (breaches).

- The corridor of routes 1 and 8 at Elgin St / Lygon St

  - In the AM peak there were four breaches, one in each rolling hour. There were two observed in 2013. The average arrival load increased by 11.6 per cent year on year

  - In the PM peak there were three breaches and one within ten per cent of desired standards. The average arrival load increased by 19.1 per cent year on year

- The Route 57 tram heading south bound, at Errol St/Victoria St, recorded no rolling hour breaches in the peaks. In May 2013 there were two rolling hour breaches in the AM peak. However the average load decreased in the AM and PM peaks by 6.6 per cent and 4.9 per cent respectively.

- The Route 55 tram heading south bound towards the CBD was observed with six breaches over two locations in the AM peak

  - The breaches at Flemington Rd / Abbotsford St increased from two to three. They were observed in the same rolling hours were there were previous breaches or within 10 per cent of a breach.

  - Similarly for breaches at Peel St / Victoria St, the number of breaches increased from two to three in the AM peak. The average load increased by 16.3 per cent year on year.

For more details of changes from 2013 to 2014 at locations where breaches occurred see the charts in Appendix 5.

## Rolling hour average loads above desired standards – Cordon locations

There are twelve cordon locations that have been consistently surveyed since October 2011. These locations represent entry or exit points for all routes through the CBD.

Table 3  below shows where breaching at cordon locations were observed in either May 2013, or May 2014.

Table 3 Breaches observed at Cordon locations in May 2013 or 2014

| Location | Routes | May 2013 AM | May 2013 PM | May 2014 AM | May 2014 PM |
| --- | --- | --- | --- | --- | --- |
| Bourke St / Spring St | 86, 96 | -- | -- | -- | -- |
| Clarendon St (Crown Casino) | 96, 109, 112 | 1 | -- | -- | -- |
| Collins St West End (Southern Cross Station) | 11, 31, 48 | -- | 3 | -- | 1 |
| Docklands (LaTrobe St West End) | 30, 86, City Circle | -- | -- | -- | -- |
| Elgin St / Lygon St | 1, 8 | 2 | 1 | 4 | 3 |
| Errol St / Victoria St | 50, 57 | 2 | -- | -- | -- |
| Flinders St / Russell St | 70, 75 | -- | -- | -- | -- |
| Haymarket (Elizabeth St) | 19, 59 | -- | -- | 2 | -- |
| Peel St / Victoria St | 55 | 2 | -- | 3 | -- |
| Queensbridge St (Casino East) | 55 | -- | -- | -- | -- |
| St Vincents Plaza | 24, 30, 109, 112 | -- | -- | 2 | -- |
| Swanston St / Flinders St (Federation Sq) | 1,3,5,6,8,16,64,67,72 | 2 | -- | -- | 3 |
| Wellington Pde / Jolimont Rd | 48, 75 | 5 | 4 | 4 | 4 |
| Subtotal |  | 14 | 8 | 15 | 11 |
| TOTAL |  | 22 |  | 26 |  |
| * 'Swanston St / Flinders St (Federation Sq)' includes trams travelling in both directions during both peaks. |  |  |  |  |  |

### Key Results

- In May 2014, 26 rolling hour average loads above desired standards were observed at seven of the cordon locations, the same number as May 2013.

- In May 2014, 15 rolling hour average loads above desired standards were observed in the AM peak, an increase of one from May 2013.

- In May 2014, 11 rolling hour average loads above desired standards were observed in the PM peak, an increase of three from May 2013.

## Average Daily Observed Loads

The cordon locations enable an estimate of the number of people arriving in the CBD by tram in the morning and the number of people departing the CBD by tram during the evening.

Table 4 Average Daily Arrival Load at Locations surrounding CBD in the morning (7:01-10:00am)

| Location | Routes | Direction | Average Daily Arrival Load May 2013 | Average Daily Arrival Load May 2014 | Change in Load from May 2013 | % Change in Load from May 2013 |
| --- | --- | --- | --- | --- | --- | --- |
| Bourke St / Spring St | 86, 96 | West | 3,791 | 4,049 | 258 | 6.8 |
| Clarendon St (Crown Casino) | 96, 109, 112 | North | 5,006 | 5,443 | 437 | 8.7 |
| Docklands (LaTrobe St West End) | 30, 86, City Circle | East | 485 | 527 | 42 | 8.7 |
| Elgin St / Lygon St | 1, 8 | South | 2,810 | 3,137 | 327 | 11.6 |
| Errol St / Victoria St | 50, 57 | South | 1,168 | 1,091 | -77 | -6.6 |
| Flinders St / Russell St | 70, 75 | West | 1,528 | 1,710 | 183 | 11.9 |
| Haymarket (Elizabeth St) | 19, 59 | South | 4,302 | 5,046 | 744 | 17.3 |
| Peel St / Victoria St | 55 | South | 2,107 | 2,451 | 344 | 16.3 |
| Queensbridge St (Casino East) | 55 | North | 1,201 | 1,314 | 113 | 9.4 |
| St Vincents Plaza | 24, 30, 109, 112 | West | 4,720 | 4,580 | -141 | -3.0 |
| Swanston St / Flinders St (Federation Sq) | 1,3,5,6,8,16,64,67,72 | North | 5,883 | 6,521 | 639 | 10.9 |
| Wellington Pde / Jolimont Rd | 48 | West | 1,600 | 1,638 | 38 | 2.4 |
| Total AM Arrival Load |  |  | 34,116 | 36,980 | 2,864 | 8.4 |

Table 5 Average Daily Departure Load at Locations surrounding CBD in the evening (3.01 - 7:00pm)

| Location |  | Routes |  | Direction |  | Average Daily Departure Load May 2013 |  | Average Daily Departure Load May 2014 |  | Change in Load from May 2013 |  | % Change in Load from May 2013 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bourke St / Spring St | 86, 96 |  | East |  | 4,434 |  | 5,323 |  | 888 |  | 20.0 |  |
| Clarendon St (Crown Casino) | 96, 109, 112 |  | South |  | 5,843 |  | 6,048 |  | 204 |  | 3.5 |  |
| Docklands (LaTrobe St West End) | 30, 86, City Circle |  | West |  | 1,005 |  | 1,063 |  | 58 |  | 5.8 |  |
| Elgin St / Lygon St | 1, 8 |  | North |  | 3,085 |  | 3,674 |  | 588 |  | 19.1 |  |
| Errol St / Victoria St | 50, 57 |  | North |  | 1,300 |  | 1,236 |  | -64 |  | -4.9 |  |
| Flinders St / Russell St | 70, 75 |  | East |  | 2,434 |  | 2,671 |  | 237 |  | 9.7 |  |
| Haymarket (Elizabeth St) | 19, 59 |  | North |  | 5,647 |  | 5,512 |  | -135 |  | -2.4 |  |
| Peel St / Victoria St | 55 |  | North |  | 1,983 |  | 2,085 |  | 103 |  | 5.2 |  |
| Queensbridge St (Casino East) | 55 |  | South |  | 782 |  | 892 |  | 110 |  | 14.0 |  |
| St Vincents Plaza | 24, 30, 109, 112 |  | East |  | 5,184 |  | 5,211 |  | 27 |  | 0.5 |  |
| Swanston St / Flinders St (Federation Sq) | 1,3,5,6,8,16,64,67,72 |  | South |  | 8,407 |  | 9,173 |  | 766 |  | 9.1 |  |
| Wellington Pde / Jolimont Rd | 48 |  | East |  | 1,622 |  | 1,866 |  | 244 |  | 15.0 |  |
| Total PM Departure Load |  |  |  |  | 40,722 |  | 43,690 |  | 2,968 |  | 7.3 |  |

### Key Results

- In May 2014, the observed average daily arrival load at locations surrounding the CBD (cordons) in the morning was 36,980 representing a 8.4 per cent increase (an increase of 2,864) from May 2013

- Similarly, the observed average daily departure load at locations surrounding the CBD in the evening was 43,690, an increase of 7.0 per cent (an increase of 2,968).

- The Swanston St / Flinders St location continues to be the most heavily loaded, representing 17.6 per cent of passengers arriving into the CBD in the morning, and 11.9 per cent of passengers departing the CBD in the evening.

- Map of survey locations

- List of survey locations

| Routes | Stop | Location | Tram Tracker ID |
| --- | --- | --- | --- |
| 86,95,96 | 9 | Bourke St / Spring St (To Bundoora RMIT/East Brunswick/Melbourne Museum) | 3309 |
| 86,95,96 | 9 | Bourke St / Spring St (To Docklands/St Kilda Beach/City-Latrobe St) | 3209 |
| 96, 112, 109 | 124A | Clarendon St (Crown Casino) (To East Brunswick/West Preston/Box Hill) | 3357 |
| 96, 112, 109 | 124A | Clarendon St (Crown Casino) (To St Kilda Beach, St Kilda (Fitzroy St)/Port Melbourne) | 3257 |
| 11,31,48 | D14 | Collins St West End (Southern Cross Station) (To North Balwyn/St Vincents) | 3400 |
| 11,31,48 | D14 | Collins St West End (Southern Cross Station) (To Victoria Harbour) | 3500 |
| 5, 64 | 33 | Dandenong Rd / Hornby St (To Malvern/East Brighton) | 2103 |
| 5, 64 | 33 | Dandenong Rd / Hornby St (To City/University) | 1103 |
| 55 | 33 | Dawson St/Daly St (Brunswick West) (To Domain Interchange) | 1263 |
| 55 | 33 | Dawson St/South Daly St (Brunswick) (To West Coburg) | 2263 |
| 3,5,6,8,16,55*,64,67,72 | 20 | Domain Interchange (St Kilda Rd - From City) | 3020 |
| 3,5,6,8,16,55*,64,67,72 | 20 | Domain Interchange (St Kilda Rd - To City) | 3120 |
| 1,8 | 112 | Elgin St / Lygon St (To East Coburg/Moreland) | 2224 |
| 67 | 44 | Elsternwick Railway Station/Glenhuntly Rd (Elsternwick) (To Carnegie) | 2044 |
| 67 | 44 | Elsternwick Railway Station/Glenhuntly Rd (Elsternwick) (To Melbourne University) | 1044 |
| 57 | 11 | Errol St / Victoria St (To City) | 3921 |
| 55, 59 | 19 | Flemington Rd / Abbotsford St (To City) | 3177 |
| 55, 59 | 19 | Flemington Rd / Abbotsford St (To West Coburg/Airport West) | 3074 |
| 35,70,75 | 6 | Flinders St / Russell St (To Docklands Drive/City-Latrobe St/Harbour Esplanade) | 3706 |
| 35,70,75 | 6 | Flinders St / Russell St (To Wattle Park/Vermont South) | 3606 |
| 35,70, 75 | D6 | Flinders St West End ( To Docklands) | 3700 |
| 35,70, 75 | D6 | Flinders St West End ( To Wattle Park) | 3600 |
| 55 | 29 | Grantham St / Brunswick Rd (To Domain Interchange) | 1259 |
| 55 | 29 | Grantham St / Brunswick Rd (To West Coburg) | 2259 |
| 19,59 | 9 | Haymarket (Elizabeth St/Pelham St) (To City) | 3909 |
| 19,59 | 9 | Haymarket (Elizabeth St/Pelham St) (To North Coburg/Airport West) | 3809 |
| 24,30,35,86,95 | D1 | Latrobe St West End (Etihad Stadium) (To Bundoora RMIT/St Vincents Plaza/North Balwyn) | 3550 |
| 24,30,35,86,95 | D1 | Latrobe St West End (Etihad Stadium) (To Docklands) | 3650 |
| 1,8 | 112 | Lygon St/Elgin St (To Sth Melb Beach/Toorak) | 1224 |
| 55 | 10 | Peel St / Victoria St (To Domain Interchange) | 3169 |
| 55 | 10 | Peel St / Victoria St (To West Coburg) | 3067 |
| 55 | 112 | Queensbridge St (Casino East) (To Domain Interchange) | 3159 |
| 55 | 112 | Queensbridge St (Casino East) (To West Coburg) | 3057 |
| 19 | 10 | Royal Pde / Grattan St (Royal Melbourne Hospital) (to CBD) | 3910 |
| 19 | 10 | Royal Pde / Grattan St (Royal Melbourne Hospital) (to North Coburg) | 3810 |
| 96 | 127 | South Melbourne Market LRS (To East Brunswick) | 1532 |
| 96 | 127 | South Melbourne Market LRS (To St Kilda Beach) | 2532 |
| 3, 5, 6, 16, 64, 67, 72 | 25 | St Kilda Rd / Commercial Rd (From CBD) | 2025 |
| 3, 5, 6, 16, 64, 67, 72 | 25 | St Kilda Rd / Commercial Rd (To CBD) | 1025 |
| 3, 5, 6, 16, 64, 67 | 27 | St Kilda Rd / High St (from CBD) | 2027 |
| 3, 5, 6, 16, 64, 67 | 27 | St Kilda Rd / High St (to CBD) | 1027 |
| 16, 3, 79 | 36 | St Kilda Town Hall ( Carlisle St) (To Kew/East Malvern/North Richmond) | 2071 |
| 3, 16 | 36 | St Kilda Town Hall ( Carlisle St) (To Kew/East Malvern/North Richmond) | 2071 |
| 3, 16 | 36 | St Kilda Town Hall ( Carlisle St) (To University/St Kilda) | 1071 |
| 16, 3, 79 | 36 | St Kilda Town Hall ( Carlisle St) (To University/St Kilda) | 1071 |
| 11,24,30*,31,109,112 | 12 | St Vincent's Plaza (To Box Hill, West Preston, North Balwyn, St Vincents Plaza) | 3412 |
| 11,24,30*,31,109,112 | 12 | St Vincent's Plaza (To Port Melbourne, St Kilda - Fitzroy St, Docklands, La Trobe St - West End, Victoria Harbour) | 3512 |
| 70 | 7E | Swan St / Punt Rd (to Waterfront City) | 3727 |
| 70 | 7E | Swan St / Punt Rd (to Wattle Park) | 3627 |
| 1,3,5,6,8,16,64,67,72 | 13 | Swanston St / Flinders St (Fed Sq side) | 3013 |
| 1,3,5,6,8,16,64,67,72 | 13 | Swanston St / Flinders St (Station side) | 3113 |
| 8 | 28 | Toorak Rd/Punt Rd (South Yarra) (To Melbourne University - Moreland) | 1563 |
| 8 | 28 | Toorak Rd/Punt Rd (South Yarra) (To Toorak) | 2563 |
| 57 | 11 | Victoria St/Errol St (To West Maribynong) | 3821 |
| 57 | 8 | Victoria St/Peel St (Melbourne City) (To City) | 3918 |
| 57 | 8 | Victoria St/Peel St (Melbourne City) (To West Maribyrnong) | 3818 |
| 48,75 | 10 | Wellington Pde / Jolimont Rd (To North Balwyn/Vermont South) | 3610 |
| 48,75 | 10 | Wellington Pde / Jolimont Rd (To Victoria Habour/City - Latrobe St) | 3710 |

- Locations where routes are combined into corridors

| Aggregated Routes | Location | CBD | Direction |
| --- | --- | --- | --- |
| Routes 1 and 8 | Elgin St / Lygon St | No | South Bound |
|  |  | No | North Bound |
| Routes 3, 5, 6, 16, 64, 67, 72 | Swanston St / Flinders St (Federation Sq) | Yes | North Bound |
|  |  | Yes | South Bound |
| All CBD Routes | Bourke St / Spring St | Yes | West Bound |
|  |  | Yes | East Bound |
| All CBD Routes | Clarendon St (Crown Casino) | No | North Bound |
|  |  | No | South Bound |
| All CBD Routes | Collins St West End (Southern Cross Station) | Yes | East Bound |
|  |  | Yes | West Bound |
| All CBD Routes | Docklands (Flinders St West End) | Yes | East Bound |
|  |  | Yes | West Bound |
| All CBD Routes | Docklands (La Trobe St West End) | Yes | East Bound |
|  |  | Yes | West Bound |
| All CBD Routes | Flinders St / Russell St | Yes | West Bound |
|  |  | Yes | East Bound |
| All CBD Routes | Latrobe St / King St | Yes | East Bound |
|  |  | Yes | West Bound |

- Maximum capacities by tram type

| Tram type | Seating Capacity | Non-CBD maximum capacity | CBD maximum capacity |
| --- | --- | --- | --- |
| A-Class | 40 | 55 | 65 |
| Z-Class | 40 | 60 | 70 |
| W-Class | 46 | 65 | 75 |
| B-Class | 74 | 100 | 110 |
| Three section C-Class Citadis | 40 | 100 | 110 |
| Three section D-Class Combino | 36 | 80 | 90 |
| Five section D-Class Combino | 58 | 120 | 130 |
| B-Class Modified (Apollo) | 40 | 110 | 120 |
| Five section C2-Class Citadis | 54 | 130 | 150 |
| E-Class | 64 | 180 | 180 |

- Comparison of May 2013 and May 2014 average load and average maximum capacity rolling hour results at locations with breaches in May 2014

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Elgin St / Lygon St | 43.1 | 69.8 |
|  | 7:31-8:30 | Elgin St / Lygon St | 70.8 | 77.6 |
|  | 8:01-9:00 | Elgin St / Lygon St | 88.6 | 81.6 |
|  | 8:31-9:30 | Elgin St / Lygon St | 81.0 | 74.5 |
|  | 9:01-10:00 | Elgin St / Lygon St | 60.9 | 67.9 |
|  | 15:01-16:00 | Elgin St / Lygon St | 44.0 | 69.7 |
|  | 15:31-16:30 | Elgin St / Lygon St | 52.5 | 77.6 |
|  | 16:01-17:00 | Elgin St / Lygon St | 54.9 | 79.5 |
|  | 16:31-17:30 | Elgin St / Lygon St | 57.9 | 74.1 |
|  | 17:01-18:00 | Elgin St / Lygon St | 71.1 | 73.5 |
|  | 17:31-18:30 | Elgin St / Lygon St | 81.5 | 76.7 |
|  | 18:01-19:00 | Elgin St / Lygon St | 72.6 | 78.4 |
|  | 7:01-8:00 | Elgin St / Lygon St | 48.8 | 69.1 |
| 2014 | 7:31-8:30 | Elgin St / Lygon St | 78.6 | 76.6 |
|  | 8:01-9:00 | Elgin St / Lygon St | 96.3 | 77.6 |
|  | 8:31-9:30 | Elgin St / Lygon St | 83.3 | 68.8 |
|  | 9:01-10:00 | Elgin St / Lygon St | 67.7 | 66.3 |
|  | 15:01-16:00 | Elgin St / Lygon St | 52.0 | 67.8 |
|  | 15:31-16:30 | Elgin St / Lygon St | 54.1 | 74.7 |
|  | 16:01-17:00 | Elgin St / Lygon St | 52.6 | 73.5 |
|  | 16:31-17:30 | Elgin St / Lygon St | 66.8 | 71.8 |
|  | 17:01-18:00 | Elgin St / Lygon St | 78.7 | 75.3 |
|  | 17:31-18:30 | Elgin St / Lygon St | 82.7 | 73.3 |
|  | 18:01-19:00 | Elgin St / Lygon St | 83.0 | 72.3 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Errol St / Victoria St | 43.7 | 60.0 |
|  | 7:31-8:30 | Errol St / Victoria St | 60.1 | 60.0 |
|  | 8:01-9:00 | Errol St / Victoria St | 63.3 | 60.0 |
|  | 8:31-9:30 | Errol St / Victoria St | 51.6 | 60.0 |
|  | 9:01-10:00 | Errol St / Victoria St | 40.4 | 60.0 |
|  | 15:01-16:00 | Errol St / Victoria St | 31.9 | 60.0 |
|  | 15:31-16:30 | Errol St / Victoria St | 30.3 | 60.0 |
|  | 16:01-17:00 | Errol St / Victoria St | 31.2 | 60.0 |
|  | 16:31-17:30 | Errol St / Victoria St | 33.4 | 60.0 |
|  | 17:01-18:00 | Errol St / Victoria St | 42.5 | 60.0 |
|  | 17:31-18:30 | Errol St / Victoria St | 47.7 | 60.0 |
|  | 18:01-19:00 | Errol St / Victoria St | 50.1 | 60.0 |
| 2014 | 7:01-8:00 | Errol St / Victoria St | 40.2 | 60.0 |
|  | 7:31-8:30 | Errol St / Victoria St | 51.2 | 60.0 |
|  | 8:01-9:00 | Errol St / Victoria St | 52.8 | 60.0 |
|  | 8:31-9:30 | Errol St / Victoria St | 47.3 | 60.0 |
|  | 9:01-10:00 | Errol St / Victoria St | 38.0 | 60.0 |
|  | 15:01-16:00 | Errol St / Victoria St | 33.2 | 60.0 |
|  | 15:31-16:30 | Errol St / Victoria St | 31.8 | 60.0 |
|  | 16:01-17:00 | Errol St / Victoria St | 32.9 | 60.0 |
|  | 16:31-17:30 | Errol St / Victoria St | 35.8 | 60.0 |
|  | 17:01-18:00 | Errol St / Victoria St | 39.5 | 60.0 |
|  | 17:31-18:30 | Errol St / Victoria St | 40.9 | 60.0 |
|  | 18:01-19:00 | Errol St / Victoria St | 45.3 | 60.0 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Peel St / Victoria St | 54.9 | 68.0 |
|  | 7:31-8:30 | Peel St / Victoria St | 73.7 | 78.0 |
|  | 8:01-9:00 | Peel St / Victoria St | 80.6 | 74.0 |
|  | 8:31-9:30 | Peel St / Victoria St | 64.9 | 64.3 |
|  | 9:01-10:00 | Peel St / Victoria St | 50.1 | 71.0 |
|  | 15:01-16:00 | Peel St / Victoria St | 34.6 | 68.0 |
|  | 15:31-16:30 | Peel St / Victoria St | 38.0 | 76.7 |
|  | 16:01-17:00 | Peel St / Victoria St | 43.9 | 80.0 |
|  | 16:31-17:30 | Peel St / Victoria St | 56.7 | 73.3 |
|  | 17:01-18:00 | Peel St / Victoria St | 67.6 | 72.2 |
|  | 17:31-18:30 | Peel St / Victoria St | 69.5 | 78.2 |
|  | 18:01-19:00 | Peel St / Victoria St | 59.1 | 83.7 |
| 2014 | 7:01-8:00 | Peel St / Victoria St | 60.8 | 68.6 |
|  | 7:31-8:30 | Peel St / Victoria St | 86.6 | 79.5 |
|  | 8:01-9:00 | Peel St / Victoria St | 86.6 | 74.5 |
|  | 8:31-9:30 | Peel St / Victoria St | 67.6 | 63.0 |
|  | 9:01-10:00 | Peel St / Victoria St | 59.4 | 71.6 |
|  | 15:01-16:00 | Peel St / Victoria St | 41.6 | 68.9 |
|  | 15:31-16:30 | Peel St / Victoria St | 41.4 | 78.3 |
|  | 16:01-17:00 | Peel St / Victoria St | 38.8 | 75.2 |
|  | 16:31-17:30 | Peel St / Victoria St | 50.4 | 69.0 |
|  | 17:01-18:00 | Peel St / Victoria St | 68.4 | 76.0 |
|  | 17:31-18:30 | Peel St / Victoria St | 72.8 | 81.6 |
|  | 18:01-19:00 | Peel St / Victoria St | 63.3 | 78.5 |

| Year | Rolling_Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | St Vincents Plaza | 47.0 | 62.0 |
|  | 7:31-8:30 | St Vincents Plaza | 58.7 | 73.3 |
|  | 8:01-9:00 | St Vincents Plaza | 70.2 | 80.6 |
|  | 8:31-9:30 | St Vincents Plaza | 66.2 | 73.6 |
|  | 9:01-10:00 | St Vincents Plaza | 57.0 | 62.1 |
|  | 15:01-16:00 | St Vincents Plaza | 46.7 | 61.1 |
|  | 15:31-16:30 | St Vincents Plaza | 56.3 | 60.0 |
|  | 16:01-17:00 | St Vincents Plaza | 54.2 | 74.3 |
|  | 16:31-17:30 | St Vincents Plaza | 56.8 | 74.7 |
|  | 17:01-18:00 | St Vincents Plaza | 63.2 | 68.0 |
|  | 17:31-18:30 | St Vincents Plaza | 62.6 | 66.3 |
|  | 18:01-19:00 | St Vincents Plaza | 62.5 | 65.8 |
| 2014 | 7:01-8:00 | St Vincents Plaza | 56.8 | 79.6 |
|  | 7:31-8:30 | St Vincents Plaza | 68.4 | 71.6 |
|  | 8:01-9:00 | St Vincents Plaza | 75.8 | 69.9 |
|  | 8:31-9:30 | St Vincents Plaza | 71.8 | 66.5 |
|  | 9:01-10:00 | St Vincents Plaza | 56.2 | 65.0 |
|  | 15:01-16:00 | St Vincents Plaza | 43.8 | 70.8 |
|  | 15:31-16:30 | St Vincents Plaza | 50.7 | 73.0 |
|  | 16:01-17:00 | St Vincents Plaza | 48.4 | 65.2 |
|  | 16:31-17:30 | St Vincents Plaza | 53.2 | 71.5 |
|  | 17:01-18:00 | St Vincents Plaza | 63.2 | 72.1 |
|  | 17:31-18:30 | St Vincents Plaza | 63.0 | 70.6 |
|  | 18:01-19:00 | St Vincents Plaza | 55.8 | 76.4 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Wellington Pde / Jolimont Rd | 57.2 | 57.1 |
|  | 7:31-8:30 | Wellington Pde / Jolimont Rd | 69.6 | 56.6 |
|  | 8:01-9:00 | Wellington Pde / Jolimont Rd | 80.5 | 61.0 |
|  | 8:31-9:30 | Wellington Pde / Jolimont Rd | 75.8 | 62.5 |
|  | 9:01-10:00 | Wellington Pde / Jolimont Rd | 62.3 | 57.4 |
|  | 15:01-16:00 | Wellington Pde / Jolimont Rd | 32.4 | 55.0 |
|  | 15:31-16:30 | Wellington Pde / Jolimont Rd | 42.1 | 55.0 |
|  | 16:01-17:00 | Wellington Pde / Jolimont Rd | 48.4 | 55.0 |
|  | 16:31-17:30 | Wellington Pde / Jolimont Rd | 57.2 | 55.0 |
|  | 17:01-18:00 | Wellington Pde / Jolimont Rd | 65.3 | 58.2 |
|  | 17:31-18:30 | Wellington Pde / Jolimont Rd | 64.7 | 58.0 |
|  | 18:01-19:00 | Wellington Pde / Jolimont Rd | 64.6 | 55.0 |
| 2014 | 7:01-8:00 | Wellington Pde / Jolimont Rd | 59.3 | 67.9 |
|  | 7:31-8:30 | Wellington Pde / Jolimont Rd | 73.4 | 72.7 |
|  | 8:01-9:00 | Wellington Pde / Jolimont Rd | 80.1 | 71.5 |
|  | 8:31-9:30 | Wellington Pde / Jolimont Rd | 72.8 | 65.4 |
|  | 9:01-10:00 | Wellington Pde / Jolimont Rd | 55.0 | 64.8 |
|  | 15:01-16:00 | Wellington Pde / Jolimont Rd | 37.3 | 75.0 |
|  | 15:31-16:30 | Wellington Pde / Jolimont Rd | 46.1 | 65.0 |
|  | 16:01-17:00 | Wellington Pde / Jolimont Rd | 55.4 | 61.4 |
|  | 16:31-17:30 | Wellington Pde / Jolimont Rd | 65.0 | 60.9 |
|  | 17:01-18:00 | Wellington Pde / Jolimont Rd | 71.8 | 72.1 |
|  | 17:31-18:30 | Wellington Pde / Jolimont Rd | 76.2 | 73.0 |
|  | 18:01-19:00 | Wellington Pde / Jolimont Rd | 76.3 | 57.0 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Wellington Pde / Jolimont Rd | 58.1 | 100.0 |
|  | 7:31-8:30 | Wellington Pde / Jolimont Rd | 79.5 | 98.0 |
|  | 8:01-9:00 | Wellington Pde / Jolimont Rd | 88.7 | 96.3 |
|  | 8:31-9:30 | Wellington Pde / Jolimont Rd | 78.4 | 95.5 |
|  | 9:01-10:00 | Wellington Pde / Jolimont Rd | 56.9 | 97.6 |
|  | 15:01-16:00 | Wellington Pde / Jolimont Rd | 41.6 | 97.5 |
|  | 15:31-16:30 | Wellington Pde / Jolimont Rd | 51.4 | 95.0 |
|  | 16:01-17:00 | Wellington Pde / Jolimont Rd | 57.1 | 97.0 |
|  | 16:31-17:30 | Wellington Pde / Jolimont Rd | 67.6 | 97.5 |
|  | 17:01-18:00 | Wellington Pde / Jolimont Rd | 79.8 | 96.1 |
|  | 17:31-18:30 | Wellington Pde / Jolimont Rd | 80.5 | 93.6 |
|  | 18:01-19:00 | Wellington Pde / Jolimont Rd | 70.7 | 93.3 |
| 2014 | 7:01-8:00 | Wellington Pde / Jolimont Rd | 65.3 | 97.6 |
|  | 7:31-8:30 | Wellington Pde / Jolimont Rd | 94.6 | 95.9 |
|  | 8:01-9:00 | Wellington Pde / Jolimont Rd | 102.2 | 96.3 |
|  | 8:31-9:30 | Wellington Pde / Jolimont Rd | 95.6 | 100.0 |
|  | 9:01-10:00 | Wellington Pde / Jolimont Rd | 67.6 | 97.6 |
|  | 15:01-16:00 | Wellington Pde / Jolimont Rd | 43.0 | 97.8 |
|  | 15:31-16:30 | Wellington Pde / Jolimont Rd | 64.8 | 100.0 |
|  | 16:01-17:00 | Wellington Pde / Jolimont Rd | 67.4 | 95.3 |
|  | 16:31-17:30 | Wellington Pde / Jolimont Rd | 71.2 | 94.7 |
|  | 17:01-18:00 | Wellington Pde / Jolimont Rd | 96.1 | 94.7 |
|  | 17:31-18:30 | Wellington Pde / Jolimont Rd | 88.0 | 91.8 |
|  | 18:01-19:00 | Wellington Pde / Jolimont Rd | 71.3 | 95.3 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Haymarket (Elizabeth St) | 69.0 | 100.0 |
|  | 7:31-8:30 | Haymarket (Elizabeth St) | 75.2 | 100.0 |
|  | 8:01-9:00 | Haymarket (Elizabeth St) | 77.7 | 100.0 |
|  | 8:31-9:30 | Haymarket (Elizabeth St) | 77.6 | 100.0 |
|  | 9:01-10:00 | Haymarket (Elizabeth St) | 72.0 | 100.0 |
|  | 15:01-16:00 | Haymarket (Elizabeth St) | 60.1 | 100.0 |
|  | 15:31-16:30 | Haymarket (Elizabeth St) | 58.8 | 100.0 |
|  | 16:01-17:00 | Haymarket (Elizabeth St) | 65.1 | 100.0 |
|  | 16:31-17:30 | Haymarket (Elizabeth St) | 80.4 | 100.0 |
|  | 17:01-18:00 | Haymarket (Elizabeth St) | 86.1 | 100.0 |
|  | 17:31-18:30 | Haymarket (Elizabeth St) | 79.8 | 100.0 |
|  | 18:01-19:00 | Haymarket (Elizabeth St) | 83.3 | 100.0 |
| 2014 | 7:01-8:00 | Haymarket (Elizabeth St) | 68.2 | 100.0 |
|  | 7:31-8:30 | Haymarket (Elizabeth St) | 86.9 | 100.0 |
|  | 8:01-9:00 | Haymarket (Elizabeth St) | 97.4 | 100.0 |
|  | 8:31-9:30 | Haymarket (Elizabeth St) | 105.7 | 98.7 |
|  | 9:01-10:00 | Haymarket (Elizabeth St) | 100.4 | 98.4 |
|  | 15:01-16:00 | Haymarket (Elizabeth St) | 62.9 | 100.0 |
|  | 15:31-16:30 | Haymarket (Elizabeth St) | 57.5 | 100.0 |
|  | 16:01-17:00 | Haymarket (Elizabeth St) | 58.4 | 100.0 |
|  | 16:31-17:30 | Haymarket (Elizabeth St) | 73.7 | 100.0 |
|  | 17:01-18:00 | Haymarket (Elizabeth St) | 92.6 | 100.0 |
|  | 17:31-18:30 | Haymarket (Elizabeth St) | 86.7 | 100.0 |
|  | 18:01-19:00 | Haymarket (Elizabeth St) | 80.0 | 100.0 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | St Kilda Rd / Commercial Rd | 35.7 | 72.2 |
|  | 7:31-8:30 | St Kilda Rd / Commercial Rd | 61.0 | 69.7 |
|  | 8:01-9:00 | St Kilda Rd / Commercial Rd | 71.9 | 63.4 |
|  | 8:31-9:30 | St Kilda Rd / Commercial Rd | 53.9 | 63.1 |
|  | 9:01-10:00 | St Kilda Rd / Commercial Rd | 35.7 | 65.9 |
|  | 15:01-16:00 | St Kilda Rd / Commercial Rd | 28.8 | 68.2 |
|  | 15:31-16:30 | St Kilda Rd / Commercial Rd | 35.6 | 66.6 |
|  | 16:01-17:00 | St Kilda Rd / Commercial Rd | 37.5 | 64.9 |
|  | 16:31-17:30 | St Kilda Rd / Commercial Rd | 46.2 | 63.8 |
|  | 17:01-18:00 | St Kilda Rd / Commercial Rd | 59.7 | 67.8 |
|  | 17:31-18:30 | St Kilda Rd / Commercial Rd | 57.7 | 71.5 |
|  | 18:01-19:00 | St Kilda Rd / Commercial Rd | 48.2 | 70.0 |
| 2014 | 7:01-8:00 | St Kilda Rd / Commercial Rd | 42.5 | 72.4 |
|  | 7:31-8:30 | St Kilda Rd / Commercial Rd | 67.6 | 70.2 |
|  | 8:01-9:00 | St Kilda Rd / Commercial Rd | 70.8 | 62.6 |
|  | 8:31-9:30 | St Kilda Rd / Commercial Rd | 53.8 | 62.3 |
|  | 9:01-10:00 | St Kilda Rd / Commercial Rd | 36.8 | 65.8 |
|  | 15:01-16:00 | St Kilda Rd / Commercial Rd | 30.5 | 68.0 |
|  | 15:31-16:30 | St Kilda Rd / Commercial Rd | 34.4 | 65.2 |
|  | 16:01-17:00 | St Kilda Rd / Commercial Rd | 37.2 | 64.9 |
|  | 16:31-17:30 | St Kilda Rd / Commercial Rd | 47.4 | 64.9 |
|  | 17:01-18:00 | St Kilda Rd / Commercial Rd | 59.9 | 67.2 |
|  | 17:31-18:30 | St Kilda Rd / Commercial Rd | 59.0 | 69.2 |
|  | 18:01-19:00 | St Kilda Rd / Commercial Rd | 48.8 | 67.2 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | St Kilda Rd / High St | 33.7 | 72.4 |
|  | 7:31-8:30 | St Kilda Rd / High St | 61.0 | 68.9 |
|  | 8:01-9:00 | St Kilda Rd / High St | 74.4 | 63.0 |
|  | 8:31-9:30 | St Kilda Rd / High St | 56.8 | 63.5 |
|  | 9:01-10:00 | St Kilda Rd / High St | 35.2 | 66.7 |
|  | 15:01-16:00 | St Kilda Rd / High St | 30.1 | 68.1 |
|  | 15:31-16:30 | St Kilda Rd / High St | 36.4 | 68.6 |
|  | 16:01-17:00 | St Kilda Rd / High St | 37.5 | 65.3 |
|  | 16:31-17:30 | St Kilda Rd / High St | 47.6 | 63.4 |
|  | 17:01-18:00 | St Kilda Rd / High St | 60.4 | 66.4 |
|  | 17:31-18:30 | St Kilda Rd / High St | 59.2 | 71.8 |
|  | 18:01-19:00 | St Kilda Rd / High St | 49.4 | 70.9 |
| 2014 | 7:01-8:00 | St Kilda Rd / High St | 40.9 | 72.6 |
|  | 7:31-8:30 | St Kilda Rd / High St | 68.1 | 69.4 |
|  | 8:01-9:00 | St Kilda Rd / High St | 73.2 | 61.9 |
|  | 8:31-9:30 | St Kilda Rd / High St | 52.8 | 62.7 |
|  | 9:01-10:00 | St Kilda Rd / High St | 34.5 | 66.7 |
|  | 15:01-16:00 | St Kilda Rd / High St | 31.1 | 68.2 |
|  | 15:31-16:30 | St Kilda Rd / High St | 37.1 | 66.4 |
|  | 16:01-17:00 | St Kilda Rd / High St | 37.9 | 64.8 |
|  | 16:31-17:30 | St Kilda Rd / High St | 47.7 | 65.0 |
|  | 17:01-18:00 | St Kilda Rd / High St | 60.1 | 67.3 |
|  | 17:31-18:30 | St Kilda Rd / High St | 56.5 | 69.2 |
|  | 18:01-19:00 | St Kilda Rd / High St | 45.6 | 66.9 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Dandenong Rd / Hornby St | 41.7 | 69.3 |
|  | 7:31-8:30 | Dandenong Rd / Hornby St | 68.7 | 65.6 |
|  | 8:01-9:00 | Dandenong Rd / Hornby St | 73.2 | 64.1 |
|  | 8:31-9:30 | Dandenong Rd / Hornby St | 50.1 | 66.3 |
|  | 9:01-10:00 | Dandenong Rd / Hornby St | 32.8 | 73.4 |
|  | 15:01-16:00 | Dandenong Rd / Hornby St | 33.7 | 66.0 |
|  | 15:31-16:30 | Dandenong Rd / Hornby St | 42.2 | 67.2 |
|  | 16:01-17:00 | Dandenong Rd / Hornby St | 43.3 | 65.5 |
|  | 16:31-17:30 | Dandenong Rd / Hornby St | 49.3 | 66.2 |
|  | 17:01-18:00 | Dandenong Rd / Hornby St | 60.1 | 65.5 |
|  | 17:31-18:30 | Dandenong Rd / Hornby St | 61.8 | 69.7 |
|  | 18:01-19:00 | Dandenong Rd / Hornby St | 54.3 | 70.8 |
| 2014 | 7:01-8:00 | Dandenong Rd / Hornby St | 44.2 | 72.7 |
|  | 7:31-8:30 | Dandenong Rd / Hornby St | 69.4 | 68.0 |
|  | 8:01-9:00 | Dandenong Rd / Hornby St | 69.7 | 61.3 |
|  | 8:31-9:30 | Dandenong Rd / Hornby St | 41.1 | 65.3 |
|  | 9:01-10:00 | Dandenong Rd / Hornby St | 28.4 | 72.8 |
|  | 15:01-16:00 | Dandenong Rd / Hornby St | 26.6 | 66.4 |
|  | 15:31-16:30 | Dandenong Rd / Hornby St | 34.1 | 64.0 |
|  | 16:01-17:00 | Dandenong Rd / Hornby St | 37.8 | 66.3 |
|  | 16:31-17:30 | Dandenong Rd / Hornby St | 45.1 | 68.2 |
|  | 17:01-18:00 | Dandenong Rd / Hornby St | 60.8 | 65.0 |
|  | 17:31-18:30 | Dandenong Rd / Hornby St | 67.7 | 64.4 |
|  | 18:01-19:00 | Dandenong Rd / Hornby St | 53.7 | 65.0 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Flemington Rd / Abbotsford St | 52.8 | 68.2 |
|  | 7:31-8:30 | Flemington Rd / Abbotsford St | 75.2 | 78.1 |
|  | 8:01-9:00 | Flemington Rd / Abbotsford St | 80.8 | 72.4 |
|  | 8:31-9:30 | Flemington Rd / Abbotsford St | 67.1 | 65.4 |
|  | 9:01-10:00 | Flemington Rd / Abbotsford St | 56.1 | 75.7 |
|  | 15:01-16:00 | Flemington Rd / Abbotsford St | 51.6 | 68.4 |
|  | 15:31-16:30 | Flemington Rd / Abbotsford St | 54.8 | 71.4 |
|  | 16:01-17:00 | Flemington Rd / Abbotsford St | 52.0 | 78.5 |
|  | 16:31-17:30 | Flemington Rd / Abbotsford St | 65.2 | 77.6 |
|  | 17:01-18:00 | Flemington Rd / Abbotsford St | 73.4 | 71.7 |
|  | 17:31-18:30 | Flemington Rd / Abbotsford St | 72.7 | 75.8 |
|  | 18:01-19:00 | Flemington Rd / Abbotsford St | 68.8 | 85.7 |
| 2014 | 7:01-8:00 | Flemington Rd / Abbotsford St | 65.9 | 71.3 |
|  | 7:31-8:30 | Flemington Rd / Abbotsford St | 87.2 | 80.5 |
|  | 8:01-9:00 | Flemington Rd / Abbotsford St | 81.1 | 71.6 |
|  | 8:31-9:30 | Flemington Rd / Abbotsford St | 65.6 | 64.2 |
|  | 9:01-10:00 | Flemington Rd / Abbotsford St | 60.2 | 75.4 |
|  | 15:01-16:00 | Flemington Rd / Abbotsford St | 55.8 | 68.9 |
|  | 15:31-16:30 | Flemington Rd / Abbotsford St | 53.5 | 72.2 |
|  | 16:01-17:00 | Flemington Rd / Abbotsford St | 56.7 | 74.7 |
|  | 16:31-17:30 | Flemington Rd / Abbotsford St | 67.5 | 73.8 |
|  | 17:01-18:00 | Flemington Rd / Abbotsford St | 72.8 | 72.2 |
|  | 17:31-18:30 | Flemington Rd / Abbotsford St | 76.1 | 79.5 |
|  | 18:01-19:00 | Flemington Rd / Abbotsford St | 68.3 | 82.7 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Flemington Rd / Abbotsford St | 73.7 | 100.0 |
|  | 7:31-8:30 | Flemington Rd / Abbotsford St | 88.0 | 100.0 |
|  | 8:01-9:00 | Flemington Rd / Abbotsford St | 93.9 | 100.0 |
|  | 8:31-9:30 | Flemington Rd / Abbotsford St | 98.0 | 100.0 |
|  | 9:01-10:00 | Flemington Rd / Abbotsford St | 93.6 | 100.0 |
|  | 15:01-16:00 | Flemington Rd / Abbotsford St | 61.0 | 100.0 |
|  | 15:31-16:30 | Flemington Rd / Abbotsford St | 66.3 | 100.0 |
|  | 16:01-17:00 | Flemington Rd / Abbotsford St | 72.1 | 100.0 |
|  | 16:31-17:30 | Flemington Rd / Abbotsford St | 77.1 | 100.0 |
|  | 17:01-18:00 | Flemington Rd / Abbotsford St | 84.6 | 100.0 |
|  | 17:31-18:30 | Flemington Rd / Abbotsford St | 87.5 | 100.0 |
|  | 18:01-19:00 | Flemington Rd / Abbotsford St | 78.4 | 100.0 |
| 2014 | 7:01-8:00 | Flemington Rd / Abbotsford St | 87.4 | 100.0 |
|  | 7:31-8:30 | Flemington Rd / Abbotsford St | 101.4 | 100.0 |
|  | 8:01-9:00 | Flemington Rd / Abbotsford St | 108.2 | 98.9 |
|  | 8:31-9:30 | Flemington Rd / Abbotsford St | 108.1 | 98.6 |
|  | 9:01-10:00 | Flemington Rd / Abbotsford St | 95.3 | 100.0 |
|  | 15:01-16:00 | Flemington Rd / Abbotsford St | 83.6 | 100.0 |
|  | 15:31-16:30 | Flemington Rd / Abbotsford St | 77.7 | 100.0 |
|  | 16:01-17:00 | Flemington Rd / Abbotsford St | 76.3 | 100.0 |
|  | 16:31-17:30 | Flemington Rd / Abbotsford St | 97.7 | 100.0 |
|  | 17:01-18:00 | Flemington Rd / Abbotsford St | 110.6 | 100.0 |
|  | 17:31-18:30 | Flemington Rd / Abbotsford St | 97.2 | 100.0 |
|  | 18:01-19:00 | Flemington Rd / Abbotsford St | 89.0 | 100.0 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Grantham St / Brunswick Rd | 61.8 | 71.7 |
|  | 7:31-8:30 | Grantham St / Brunswick Rd | 80.2 | 77.8 |
|  | 8:01-9:00 | Grantham St / Brunswick Rd | 76.0 | 69.2 |
|  | 8:31-9:30 | Grantham St / Brunswick Rd | 61.0 | 65.0 |
|  | 9:01-10:00 | Grantham St / Brunswick Rd | 50.1 | 73.3 |
|  | 15:01-16:00 | Grantham St / Brunswick Rd | 41.4 | 68.0 |
|  | 15:31-16:30 | Grantham St / Brunswick Rd | 46.2 | 69.5 |
|  | 16:01-17:00 | Grantham St / Brunswick Rd | 47.2 | 77.1 |
|  | 16:31-17:30 | Grantham St / Brunswick Rd | 51.1 | 79.4 |
|  | 17:01-18:00 | Grantham St / Brunswick Rd | 55.9 | 73.3 |
|  | 17:31-18:30 | Grantham St / Brunswick Rd | 62.9 | 75.4 |
|  | 18:01-19:00 | Grantham St / Brunswick Rd | 65.4 | 80.7 |
| 2014 | 7:01-8:00 | Grantham St / Brunswick Rd | 61.0 | 72.6 |
|  | 7:31-8:30 | Grantham St / Brunswick Rd | 74.1 | 76.7 |
|  | 8:01-9:00 | Grantham St / Brunswick Rd | 64.2 | 67.1 |
|  | 8:31-9:30 | Grantham St / Brunswick Rd | 56.2 | 66.9 |
|  | 9:01-10:00 | Grantham St / Brunswick Rd | 55.1 | 79.1 |
|  | 15:01-16:00 | Grantham St / Brunswick Rd | 55.8 | 68.0 |
|  | 15:31-16:30 | Grantham St / Brunswick Rd | 57.8 | 70.4 |
|  | 16:01-17:00 | Grantham St / Brunswick Rd | 60.7 | 78.3 |
|  | 16:31-17:30 | Grantham St / Brunswick Rd | 61.4 | 77.3 |
|  | 17:01-18:00 | Grantham St / Brunswick Rd | 64.7 | 71.6 |
|  | 17:31-18:30 | Grantham St / Brunswick Rd | 68.9 | 76.8 |
|  | 18:01-19:00 | Grantham St / Brunswick Rd | 66.1 | 78.1 |

| Year | Rolling_Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Swan St / Punt Rd | 32.1 | 59.7 |
|  | 7:31-8:30 | Swan St / Punt Rd | 39.9 | 55.0 |
|  | 8:01-9:00 | Swan St / Punt Rd | 37.2 | 59.7 |
|  | 8:31-9:30 | Swan St / Punt Rd | 28.5 | 64.0 |
|  | 9:01-10:00 | Swan St / Punt Rd | 23.9 | 60.2 |
|  | 15:01-16:00 | Swan St / Punt Rd | 29.2 | 55.0 |
|  | 15:31-16:30 | Swan St / Punt Rd | 28.4 | 55.0 |
|  | 16:01-17:00 | Swan St / Punt Rd | 32.3 | 57.5 |
|  | 16:31-17:30 | Swan St / Punt Rd | 34.7 | 60.0 |
|  | 17:01-18:00 | Swan St / Punt Rd | 47.2 | 59.5 |
|  | 17:31-18:30 | Swan St / Punt Rd | 57.5 | 57.3 |
|  | 18:01-19:00 | Swan St / Punt Rd | 49.5 | 55.0 |
| 2014 | 7:01-8:00 | Swan St / Punt Rd | 32.9 | 55.0 |
|  | 7:31-8:30 | Swan St / Punt Rd | 39.0 | 55.0 |
|  | 8:01-9:00 | Swan St / Punt Rd | 36.4 | 55.0 |
|  | 8:31-9:30 | Swan St / Punt Rd | 33.3 | 57.1 |
|  | 9:01-10:00 | Swan St / Punt Rd | 28.9 | 57.4 |
|  | 15:01-16:00 | Swan St / Punt Rd | 22.9 | 55.0 |
|  | 15:31-16:30 | Swan St / Punt Rd | 27.2 | 57.5 |
|  | 16:01-17:00 | Swan St / Punt Rd | 34.2 | 57.5 |
|  | 16:31-17:30 | Swan St / Punt Rd | 41.5 | 57.5 |
|  | 17:01-18:00 | Swan St / Punt Rd | 46.9 | 59.3 |
|  | 17:31-18:30 | Swan St / Punt Rd | 42.5 | 60.4 |
|  | 18:01-19:00 | Swan St / Punt Rd | 35.7 | 59.5 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Collins St West End (Southern Cross Station) | 49.1 | 65.0 |
|  | 7:31-8:30 | Collins St West End (Southern Cross Station) | 58.3 | 76.4 |
|  | 8:01-9:00 | Collins St West End (Southern Cross Station) | 65.0 | 79.3 |
|  | 8:31-9:30 | Collins St West End (Southern Cross Station) | 62.9 | 78.4 |
|  | 9:01-10:00 | Collins St West End (Southern Cross Station) | 51.4 | 80.3 |
|  | 15:01-16:00 | Collins St West End (Southern Cross Station) | 47.3 | 72.5 |
|  | 15:31-16:30 | Collins St West End (Southern Cross Station) | 64.5 | 68.5 |
|  | 16:01-17:00 | Collins St West End (Southern Cross Station) | 59.9 | 73.7 |
|  | 16:31-17:30 | Collins St West End (Southern Cross Station) | 65.0 | 71.7 |
|  | 17:01-18:00 | Collins St West End (Southern Cross Station) | 76.9 | 68.3 |
|  | 17:31-18:30 | Collins St West End (Southern Cross Station) | 81.3 | 67.4 |
|  | 18:01-19:00 | Collins St West End (Southern Cross Station) | 68.1 | 65.0 |
| 2014 | 7:01-8:00 | Collins St West End (Southern Cross Station) | 40.8 | 83.0 |
|  | 7:31-8:30 | Collins St West End (Southern Cross Station) | 52.1 | 85.1 |
|  | 8:01-9:00 | Collins St West End (Southern Cross Station) | 65.0 | 84.1 |
|  | 8:31-9:30 | Collins St West End (Southern Cross Station) | 58.6 | 77.3 |
|  | 9:01-10:00 | Collins St West End (Southern Cross Station) | 46.1 | 71.1 |
|  | 15:01-16:00 | Collins St West End (Southern Cross Station) | 53.5 | 80.9 |
|  | 15:31-16:30 | Collins St West End (Southern Cross Station) | 57.7 | 71.9 |
|  | 16:01-17:00 | Collins St West End (Southern Cross Station) | 57.6 | 77.7 |
|  | 16:31-17:30 | Collins St West End (Southern Cross Station) | 71.0 | 76.6 |
|  | 17:01-18:00 | Collins St West End (Southern Cross Station) | 83.1 | 81.6 |
|  | 17:31-18:30 | Collins St West End (Southern Cross Station) | 75.4 | 80.9 |
|  | 18:01-19:00 | Collins St West End (Southern Cross Station) | 58.5 | 73.5 |

| Year | Rolling_Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Swanston St / Flinders St (Federation Sq) | 51.1 | 73.7 |
|  | 7:31-8:30 | Swanston St / Flinders St (Federation Sq) | 82.2 | 79.7 |
|  | 8:01-9:00 | Swanston St / Flinders St (Federation Sq) | 94.7 | 83.6 |
|  | 8:31-9:30 | Swanston St / Flinders St (Federation Sq) | 66.3 | 77.5 |
|  | 9:01-10:00 | Swanston St / Flinders St (Federation Sq) | 36.3 | 74.2 |
|  | 15:01-16:00 | Swanston St / Flinders St (Federation Sq) | 49.9 | 77.0 |
|  | 15:31-16:30 | Swanston St / Flinders St (Federation Sq) | 51.1 | 76.9 |
|  | 16:01-17:00 | Swanston St / Flinders St (Federation Sq) | 51.9 | 76.0 |
|  | 16:31-17:30 | Swanston St / Flinders St (Federation Sq) | 63.5 | 79.5 |
|  | 17:01-18:00 | Swanston St / Flinders St (Federation Sq) | 76.2 | 81.5 |
|  | 17:31-18:30 | Swanston St / Flinders St (Federation Sq) | 71.4 | 78.2 |
|  | 18:01-19:00 | Swanston St / Flinders St (Federation Sq) | 55.8 | 76.5 |
| 2014 | 7:01-8:00 | Swanston St / Flinders St (Federation Sq) | 51.1 | 74.0 |
|  | 7:31-8:30 | Swanston St / Flinders St (Federation Sq) | 76.0 | 79.8 |
|  | 8:01-9:00 | Swanston St / Flinders St (Federation Sq) | 83.6 | 84.3 |
|  | 8:31-9:30 | Swanston St / Flinders St (Federation Sq) | 62.3 | 78.6 |
|  | 9:01-10:00 | Swanston St / Flinders St (Federation Sq) | 35.8 | 73.5 |
|  | 15:01-16:00 | Swanston St / Flinders St (Federation Sq) | 51.3 | 76.4 |
|  | 15:31-16:30 | Swanston St / Flinders St (Federation Sq) | 57.7 | 78.2 |
|  | 16:01-17:00 | Swanston St / Flinders St (Federation Sq) | 59.0 | 77.2 |
|  | 16:31-17:30 | Swanston St / Flinders St (Federation Sq) | 67.9 | 79.3 |
|  | 17:01-18:00 | Swanston St / Flinders St (Federation Sq) | 81.0 | 80.0 |
|  | 17:31-18:30 | Swanston St / Flinders St (Federation Sq) | 80.1 | 77.5 |
|  | 18:01-19:00 | Swanston St / Flinders St (Federation Sq) | 60.3 | 77.4 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Swanston St / Flinders St (Federation Sq) | 41.2 | 87.1 |
|  | 7:31-8:30 | Swanston St / Flinders St (Federation Sq) | 72.4 | 82.6 |
|  | 8:01-9:00 | Swanston St / Flinders St (Federation Sq) | 88.5 | 92.0 |
|  | 8:31-9:30 | Swanston St / Flinders St (Federation Sq) | 65.4 | 92.1 |
|  | 9:01-10:00 | Swanston St / Flinders St (Federation Sq) | 42.1 | 81.5 |
|  | 15:01-16:00 | Swanston St / Flinders St (Federation Sq) | 48.0 | 93.7 |
|  | 15:31-16:30 | Swanston St / Flinders St (Federation Sq) | 48.0 | 93.3 |
|  | 16:01-17:00 | Swanston St / Flinders St (Federation Sq) | 58.0 | 85.5 |
|  | 16:31-17:30 | Swanston St / Flinders St (Federation Sq) | 71.4 | 82.5 |
|  | 17:01-18:00 | Swanston St / Flinders St (Federation Sq) | 80.1 | 89.0 |
|  | 17:31-18:30 | Swanston St / Flinders St (Federation Sq) | 81.7 | 100.6 |
|  | 18:01-19:00 | Swanston St / Flinders St (Federation Sq) | 55.1 | 96.9 |
| 2014 | 7:01-8:00 | Swanston St / Flinders St (Federation Sq) | 40.0 | 79.6 |
|  | 7:31-8:30 | Swanston St / Flinders St (Federation Sq) | 68.9 | 84.0 |
|  | 8:01-9:00 | Swanston St / Flinders St (Federation Sq) | 90.8 | 90.9 |
|  | 8:31-9:30 | Swanston St / Flinders St (Federation Sq) | 80.4 | 88.3 |
|  | 9:01-10:00 | Swanston St / Flinders St (Federation Sq) | 43.1 | 74.7 |
|  | 15:01-16:00 | Swanston St / Flinders St (Federation Sq) | 41.5 | 88.9 |
|  | 15:31-16:30 | Swanston St / Flinders St (Federation Sq) | 48.7 | 87.1 |
|  | 16:01-17:00 | Swanston St / Flinders St (Federation Sq) | 54.2 | 81.3 |
|  | 16:31-17:30 | Swanston St / Flinders St (Federation Sq) | 80.1 | 80.4 |
|  | 17:01-18:00 | Swanston St / Flinders St (Federation Sq) | 97.9 | 90.5 |
|  | 17:31-18:30 | Swanston St / Flinders St (Federation Sq) | 77.5 | 97.2 |
|  | 18:01-19:00 | Swanston St / Flinders St (Federation Sq) | 47.5 | 82.0 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Clarendon St (Crown Casino) | 34.7 | 76.1 |
|  | 7:31-8:30 | Clarendon St (Crown Casino) | 55.3 | 65.0 |
|  | 8:01-9:00 | Clarendon St (Crown Casino) | 65.4 | 65.0 |
|  | 8:31-9:30 | Clarendon St (Crown Casino) | 47.6 | 75.4 |
|  | 9:01-10:00 | Clarendon St (Crown Casino) | 30.1 | 87.9 |
|  | 15:01-16:00 | Clarendon St (Crown Casino) | 29.2 | 71.4 |
|  | 15:31-16:30 | Clarendon St (Crown Casino) | 33.5 | 81.8 |
|  | 16:01-17:00 | Clarendon St (Crown Casino) | 40.3 | 84.2 |
|  | 16:31-17:30 | Clarendon St (Crown Casino) | 47.0 | 74.0 |
|  | 17:01-18:00 | Clarendon St (Crown Casino) | 55.1 | 68.8 |
|  | 17:31-18:30 | Clarendon St (Crown Casino) | 61.0 | 74.0 |
|  | 18:01-19:00 | Clarendon St (Crown Casino) | 53.4 | 81.4 |
| 2014 | 7:01-8:00 | Clarendon St (Crown Casino) | 38.7 | 78.3 |
|  | 7:31-8:30 | Clarendon St (Crown Casino) | 66.2 | 77.9 |
|  | 8:01-9:00 | Clarendon St (Crown Casino) | 86.3 | 88.1 |
|  | 8:31-9:30 | Clarendon St (Crown Casino) | 62.1 | 71.9 |
|  | 9:01-10:00 | Clarendon St (Crown Casino) | 39.8 | 82.3 |
|  | 15:01-16:00 | Clarendon St (Crown Casino) | 35.8 | 70.6 |
|  | 15:31-16:30 | Clarendon St (Crown Casino) | 38.6 | 79.3 |
|  | 16:01-17:00 | Clarendon St (Crown Casino) | 43.5 | 84.3 |
|  | 16:31-17:30 | Clarendon St (Crown Casino) | 53.1 | 81.1 |
|  | 17:01-18:00 | Clarendon St (Crown Casino) | 64.0 | 83.9 |
|  | 17:31-18:30 | Clarendon St (Crown Casino) | 58.1 | 72.8 |
|  | 18:01-19:00 | Clarendon St (Crown Casino) | 44.8 | 78.7 |

| Year | Rolling Hour | Location | Sum of Average Load | Sum of Average Maximum Capacity |
| --- | --- | --- | --- | --- |
| 2013 | 7:01-8:00 | Toorak Rd / Punt Rd | 0.0 | 0.0 |
|  | 7:31-8:30 | Toorak Rd / Punt Rd | 0.0 | 0.0 |
|  | 8:01-9:00 | Toorak Rd / Punt Rd | 0.0 | 0.0 |
|  | 8:31-9:30 | Toorak Rd / Punt Rd | 0.0 | 0.0 |
|  | 9:01-10:00 | Toorak Rd / Punt Rd | 0.0 | 0.0 |
|  | 15:01-16:00 | Toorak Rd / Punt Rd | 0.0 | 0.0 |
|  | 15:31-16:30 | Toorak Rd / Punt Rd | 0.0 | 0.0 |
|  | 16:01-17:00 | Toorak Rd / Punt Rd | 0.0 | 0.0 |
|  | 16:31-17:30 | Toorak Rd / Punt Rd | 0.0 | 0.0 |
|  | 17:01-18:00 | Toorak Rd / Punt Rd | 0.0 | 0.0 |
|  | 17:31-18:30 | Toorak Rd / Punt Rd | 0.0 | 0.0 |
|  | 18:01-19:00 | Toorak Rd / Punt Rd | 0.0 | 0.0 |
| 2014 | 7:01-8:00 | Toorak Rd / Punt Rd | 51.7 | 70.8 |
|  | 7:31-8:30 | Toorak Rd / Punt Rd | 78.9 | 68.0 |
|  | 8:01-9:00 | Toorak Rd / Punt Rd | 73.9 | 68.9 |
|  | 8:31-9:30 | Toorak Rd / Punt Rd | 53.0 | 74.6 |
|  | 9:01-10:00 | Toorak Rd / Punt Rd | 30.2 | 71.8 |
|  | 15:01-16:00 | Toorak Rd / Punt Rd | 59.7 | 70.5 |
|  | 15:31-16:30 | Toorak Rd / Punt Rd | 54.8 | 76.1 |
|  | 16:01-17:00 | Toorak Rd / Punt Rd | 41.3 | 75.0 |
|  | 16:31-17:30 | Toorak Rd / Punt Rd | 46.5 | 63.7 |
|  | 17:01-18:00 | Toorak Rd / Punt Rd | 55.5 | 71.5 |
|  | 17:31-18:30 | Toorak Rd / Punt Rd | 57.1 | 74.2 |
|  | 18:01-19:00 | Toorak Rd / Punt Rd | 53.5 | 71.2 |

- Charts of services observed in rolling hours where load is more than 90 per cent of capacity available

“Morning” refers to the time period 7.01 – 10.00am, the hours surveyed. This definition is broader than that of AM peak, which refers to the period between 7.31 am and 9.30 am.

“Evening” refers to the time period 3.01 – 7.00pm, the hours surveyed. This definition is broader than that of PM peak, which refers to the period between 3.31 pm and 6.30pm.